FINAL FORKLIFT TEST

# Name: ________________________________     Date: __________     Score: ________

- Your forklift can be driven by:

- Any employee

- An employee who has training from another facility

- Only operators specially trained by their company

- When changing directions: (forward to reverse, or reverse to forward)

- Always stop

- Slow down to about 1 mph

- Don't worry about speed; a forklift is built to take shock

- Never exceed the rated load capacity of your truck:

- Unless you add additional counterweight

- Unless your lift is under 84 inches high

- No, never

- When taking a load down a ramp:

- Drive backwards

- Drive forwards

- Reducing speed by zigzagging

- If you have to park on a ramp:

- Leave the machine in gear

- Set the brakes and block the wheels

- Set the load down, and rest the machine against it

- On a forklift, your horn:

- Makes a good device to catch the foreman's attention

- Should be sounded at intersections

- Should be sounded when you're stacking pallets

- When loading a truck trailer, you should:

- Check the trailer tires to make sure they are blocked

- Check to make sure that the tractor is attached

- Load the left side of the trailer first

- When approaching a blind corner, you should:

- Stop, and then proceed

- Slow down, sound the horn and proceed slowly

- Continue at same speed while sounding the horn

- The condition of a pallet:

- Affects load stability

- Affects load center

- Neither of the above

- The speed at which a forklift should be operated is:

- Determined by the operator

- Determined by the load, driving conditions, and plant rules

- Determined by signs posted along aisles and passageways

- When operating a fully loaded sit down riding type forklift on a ramp, you should:

- Drive up the ramp with the load in front

- Drive up the ramp with the load trailing

- It doesn't make a difference

- When parking a lift truck:

- The forks should be above eye-level to prevent an accident

- 2" above the floor

- Resting on the floor with the tips pointing down

- Safety of a forklift is the responsibility of:

- Operators

- Management

- Manufacturer

- All of the above

- After picking up a palletized load, it should be tilted:

  - As far back as possible

  - Only far enough back to stabilize the load

  - Not at all

- Forklift operator training is:

  - Required by law

  - Is only necessary for new operators

  - Is valuable even if it only prevents one accident from occurring

  - Both "a" and "c"

True or False

- Your tires should be checked and seatbelt worn every time you use the truck.

- A lift truck steers with its rear wheels and its rear end swings on turns.

- It is okay to smoke when refueling as long as you are in a restricted area.

- Start out slowly with every load.

- Loads when lifted high reduce a truck's stability.

- As long as you sit beside him, it is okay to let an unauthorized person operate your forklift.

- Never let bystanders stand beneath loads when you're stacking.

- A careful driver may hang his leg outside a machine when it is protected by a wide load.

- A good operator drives slowly on slippery and wet floors.

- One of the greatest causes of accidents is forgetting to watch overhead obstructions when lifting.

- An overhead guard is intended to offer protection from the impact of falling capacity loads.

- Never raise a fellow employee on the forks or pallets.

- The center of the load should be situated on the tips of the load forks.

- The operator does not have to inspect his truck after a mechanic has made repairs.

- Forklifts should not be parked in front of stairways and fire equipment

Final Score:

Instructor___________________________________     Date: ___________________